CP INDUSTRIES

SAFETY DATA SHEET

PREMIERE BRASS AND METAL POLISH

REVISION: 01/30/2015

| 1. PRODUCT AND COMPANY IDENTIFICATION |  |
| --- | --- |

PRODUCT IDENTIFIER: PREMIERE BRASS AND METAL POLISH

RECOMMENDED USE: SHINING METALLIC SURFACES

RECOMMENDED RESTRICTIONS: NONE IDENTIFIED

MANUFACTURED BY:
CP INDUSTRIES, LLC
560 NORTH 500 WEST
SALT LAKE CITY, UT 84116

(800) 453-4931

EMERGENCY CONTACT # (24 HOUR): INFOTRAC (800) 535-5053

| 2. HAZARDOUS IDENTIFICATION |  |
| --- | --- |

CLASSIFICATION OF THE SUBSTANCE OR MIXTURE:

GHS LABEL ELEMENTS, INCLUDING PRECAUTIONARY STATEMENTS:

REVERSIBLE EYE EFFECTS CATEGORY 2A: H319
SKIN SENSITIZER CATEGORY 1: H317
CHRONIC AQUATIC HAZARD CATEGORY 3: H412

EXCLAMATION MARK
ENVIRONMENT

SIGNAL WORD: WARNING

HAZARDS NOT OTHERWISE CLASSIFIED (HNOC) OR NOT COVERED BY GHS: NONE

HAZARD STATEMENTS:
H317: MAY CAUSE AN ALLERGIC SKIN REACTION
H319: CAUSES SERIOUS EYE IRRITATION
H400: TOXIC TO AQUATIC LIFE

PRECAUTIONARY STATEMENTS - PREVENTION:
P235: KEEP COOL
P273: AVOID RELEASE TO THE ENVIRONMENT
P281: USE PERSONAL PROTECTIVE EQUIPMENT AS REQUIRED

PRECAUTIONARY STATEMENTS - RESPONSE:
P302 + P352:
IF ON SKIN: WASH WITH SOAP AND WATER

P308 + P313:
IF EXPOSED OR CONCERNED: GET MEDICAL ADVICE/ATTENTION

P333 + P313:
IF SKIN IRRITATION OR A RASH OCCURS: GET MEDICAL ADVICE/ATTENTION

P370 + P378:
IN CASE OF FIRE:
USE CARBON DIOXIDE (CO2), DRY CHEMICAL, OR FOAM FOR EXTINCTION. DO NOT
USE A DIRECT WATER JET ON BURNING MATERIAL

P391: COLLECT SPILLAGE

| 3. COMPOSITION / INFORMATION ON INGREDIENTS |  |
| --- | --- |

SUBSTANCE(S) DESCRIPTION            CAS#             PERCENTAGE

PETROLEUM DISTILLATES               8052-41-3

AMMONIUM HYDROXIDE                  1336-21-6

PROPRIETARY SURFACTANT BLEND: N/A

NO INGREDIENTS ARE HAZARDOUS ACCORDING TO OSHA CRITERIA.

NO INGREDIENTS NO COMPONENTS NEED TO BE DISCLOSED ACCORDING TO APPLICABLE
REGULATIONS.

| 4. FIRST AID MEASURES |  |
| --- | --- |

INHALATION FIRST AID:
INHALATION IS UNLIKELY; HOWEVER, IF IT DOES OCCUR, REMOVE VICTIM TO FRESH
AIR. IF NOT BREATHING, GIVE ARTIFICIAL RESPIRATION. IF BREATHING IS
DIFFICULT, ADMINISTER OXYGEN. SEEK MEDICAL ATTENTION

SKIN CONTACT FIRST AID:
WASH OFF WITH WATER. IF IRRITATION PERSISTS SEEK MEDICAL ATTENTION.

EYE CONTACT FIRST AID:
IMMEDIATELY FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES
HOLDING EYELIDS APART TO ENSURE FLUSHING OF ENTIRE EYE SURFACE. SEEK
MEDICAL ATTENTION IF IRRITATION PERSISTS.

INGESTION FIRST AID:
IF SWALLOWED DO NOT INDUCE VOMITING. GIVE LARGE QUANTITIES OF WATER.
CONTACT A PHYSICIAN OR POISON CONTROL IMMEDIATELY. SEEK MEDICAL ATTENTION.

INDICATION OF ANY IMMEDIATE MEDICAL ATTENTION AND SPECIAL TREATMENT NEEDED:
NO DATA AVAILABLE.

| 5. FIRE FIGHTING MEASURES |  |
| --- | --- |

EXTINGUISHING MEDIA:
REGULAR FOAM, DRY CHEMICAL OR CARBON DIOXIDE. DIRECT APPLICATION OF HIGH
PRESSURE WATER STREAMS MAY SCATTER BURNING MATERIAL.

ADVICE TO FIRE FIGHTERS:
AS IN ANY FIRE, PREVENT HUMAN EXPOSURE TO FIRE, SMOKE, FUMES, OR PRODUCTS
OF COMBUSTION. EVACUATE NON-ESSENTIAL PERSONNEL FROM THE FIRE AREA.
FIREFIGHTERS SHOULD WEAR POSITIVE PRESSURE/PRESSURE DEMAND, SELF-CONTAINED
BREATHING APPARATUS AND IMPERVIOUS PROTECTIVE CLOTHING. IF POSSIBLE,
REMOVE CONTAINERS FROM THE AREA. KEEP FIRE EXPOSED CONTAINERS COOL WITH A
WATER FOG OR SPRAY TO PREVENT EXCESSIVE HEAT. HIGH PRESSURE WATER MAY
SPREAD PRODUCT FROM BROKEN CONTAINERS INCREASING CONTAMINATION OR FIRE
HAZARD.

FURTHER INFORMATION: NO DATA AVAILABLE.

| 6. ACCIDENTAL RELEASE MEASURES |  |
| --- | --- |

PERSONAL PRECAUTIONS: AVOID BREATHING VAPORS, MIST OR GAS.

PROTECTIVE EQUIPMENT AND EMERGENCY PROCEDURES:
FOR PERSONAL PROTECTION SEE SECTION 8.

ENVIRONMENTAL PRECAUTIONS:
AVOID LARGE CONCENTRATIONS OF PRODUCT IN A CONFINED AREA.

MATERIALS FOR CONTAINMENT AND CLEAN UP: BROOM AND A DUSTPAN OR SHOVEL

METHODS FOR CLEANUP CONTAINMENT:
SOAK UP SMALL SPILLS WITH AN INERT ABSORBENT MATERIAL AND DISPOSE OF IN
AN APPROPRIATE WASTE CONTAINER. LARGE SPILLS SHOULD BE DIKED, CONTAINED
AND COLLECTED FOR LATER DISPOSAL ACCORDING TO LOCAL, STATE, OR FEDERAL
REGULATIONS.

REFERENCE TO OTHER SECTIONS: SEE SECTION 13 FOR DISPOSAL PROCEDURES

| 7. HANDLING AND STORAGE |  |
| --- | --- |

PRECAUTIONS FOR SAFE HANDLING:
HARMFUL OR FATAL IF SWALLOWED. USE IN A WELL-VENTILATED AREA. WEAR
PROTECTIVE EQUIPMENT WHEN HANDLING THIS PRODUCT TO AVOID EYE AND SKIN
CONTACT. WASH THOROUGHLY AFTER HANDLING.

CONDITIONS FOR SAFE STORAGE, INCLUDING ANY INCOMPATIBILITIES:
STORE IN AN AREA INACCESSIBLE TO CHILDREN AND PETS. CLOSE CONTAINER
AFTER EACH USE. STORAGE CONTAINERS SHOULD BE STORED IN A COOL, DRY
AND WELL-VENTILATED AREA AWAY FROM STRONG OXIDIZING AGENTS.

SPECIFIC END USE(S):
APART FROM THE USES STATED IN SECTION 1, NO OTHER SPECIFIC USES ARE
STIPULATED.

| 8. EXPOSURE CONTROLS / PERSONAL PROTECTION |  |
| --- | --- |

COMPONENTS WITH WORKPLACE CONTROL PARAMETERS:
CONTAINS NO SUBSTANCES WITH OCCUPATIONAL EXPOSURE LIMIT VALUES.

RESPIRATORY PROTECTION: N/A

SKIN PROTECTION:
GLOVES SHOULD BE WORN WHEN EXCESSIVE SKIN CONTACT CANNOT BE AVOIDED.

EYE PROTECTION:
EYE PROTECTION IS RECOMMENDED IN ALL INDUSTRIAL WORK PLACES.

| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |
| --- | --- |

APPEARANCE:

FORM: LIQUID

COLOR: WHITE, OPAQUE

ODOR: CITRUS

ODOR THRESHOLD: NO DATA AVAILABLE

PH: 9-10

MELTING/FREEZING POINT:
MELTING POINT/RANGE: 801 DEG. C (1,474 DEG. F)

INITIAL BOILING POINT: 212 DEG. F (100 C)

FLASH POINT:
CLOSED CUP: 180.0 F (82.0 C)

EVAPORATION RATE: NO DATA AVAILABLE

FLAMMABILITY (SOLID, GAS): NO DATA AVAILABLE

EXPLOSIVE LIMITS: NO DATA AVAILABLE

VAPOR PRESSURE: NO DATA AVAILABLE

VAPOR DENSITY: NO DATA AVAILABLE

RELATIVE DENSITY: 1 G/CM3

WATER SOLUBILITY: PARTIALLY SOLUBLE

PARTITION COEFFICIENT N-OCTANOL/WATER: NO DATA AVAILABLE

AUTO-IGNITION TEMPERATURE: GREATER THAN 302.0 F

DECOMPOSITION TEMPERATURE: NO DATA AVAILABLE

VISCOSITY: NO DATA AVAILABLE

EXPLOSIVE PROPERTIES: NO DATA AVAILABLE

OXIDIZING PROPERTIES: NO DATA AVAILABLE

| 10. STABILITY AND REACTIVITY |  |
| --- | --- |

REACTIVITY: STABLE

CHEMICAL STABILITY: STABLE UNDER RECOMMENDED STORAGE CONDITIONS

POSSIBILITY OF HAZARDOUS REACTIONS: NO DATA AVAILABLE

CONDITIONS TO AVOID: EXPOSURE TO HIGH TEMPERATURES AND OPEN FLAMES.

INCOMPATIBLE MATERIALS: STRONG OXIDIZING AGENTS

HAZARDOUS DECOMPOSITION PRODUCTS:
TOXIC OXIDES OF CARBON AND HYDROCARBONS.
IN THE EVENT OF FIRE: SEE SECTION 5

| 11. TOXICOLOGICAL INFORMATION |  |
| --- | --- |

INFORMATION ON TOXICOLOGICAL EFFECTS:

ALTHOUGH NO SPECIFIC EXPOSURE LIMIT HAS BEEN ESTABLISHED FOR THIS PRODUCT
OSHA AND ACGIH HAVE ESTABLISHED LIMITS FOR NUISANCE DUST:
OSHA PEL/TWA TOTAL 15 MG/M3; RESPIRABLE 5 MG/M3; 8-HOUR TWA
ACGIH TLV/TWA TOTAL 10 MG/M3; 8 HOUR TWA

EYE CONTACT: CAN CAUSE MILD IRRITATION

INHALATION: NOT LIKELY

SKIN CONTACT: PROLONGED CONTINUAL EXPOSURE CAN BE IRRITATING.

INGESTION: MAY BE HARMFUL OR FATAL IF SWALLOWED..

CHRONIC: NO EVIDENCE OF ADVERSE EFFECTS FROM AVAILABLE DATA

| 12. ECOLOGICAL INFORMATION |  |
| --- | --- |

12.1 TOXICITY: NO DATA AVAILABLE

12.2 PERSISTENCE AND DEGRADABILITY: NO DATA AVAILABLE

12.3 BIOACCUMULATIVE POTENTIAL: NO DATA AVAILABLE

12.4 MOBILITY IN SOIL: NO DATA AVAILABLE

12.5 RESULTS OF PBT AND VPVB ASSESSMENT:
PBT/VPVB ASSESSMENT NOT AVAILABLE AS CHEMICAL SAFETY ASSESSMENT
NOT REQUIRED/NOT CONDUCTED

12.6 OTHER ADVERSE EFFECTS: NO DATA AVAILABLE

| 13. DISPOSAL CONSIDERATIONS |  |
| --- | --- |

PRODUCT:
DISPOSE OF IN ACCORDANCE WITH LOCAL, STATE, AND FEDERAL REGULATIONS.

CONTAMINATED PACKAGING:
CONTAINERS OF THIS MATERIAL MAY BE HAZARDOUS WHEN EMPTIED SINCE EMPTIED
CONTAINERS RETAIN PRODUCT RESIDUES (VAPOR, LIQUID, AND/OR SOLID). ALL
HAZARD PRECAUTIONS GIVEN IN THE DATA SHEET MUST BE OBSERVED. DISPOSE OF
AS UNUSED PRODUCT.

| 14. TRANSPORT INFORMATION |  |
| --- | --- |

DOT (US):
UN NUMBER: 3082
CLASS: 9
PACKING GROUP: III
PROPER SHIPPING NAME: AMMONIUM HYDROXIDE
POISON INHALATION HAZARD: NO
REPORTABLE QUANTITY (RQ): 208 LBS
MARINE POLLUTANT: YES

IMDG:
UN NUMBER: 3082
CLASS: 9
PACKING GROUP: III
PROPER SHIPPING NAME: AMMONIUM HYDROXIDE
ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S.
MARINE POLLUTANT: YES
EMS-NO: F-A, S-F

IATA:
UN NUMBER: 3082
CLASS: 9
PACKING GROUP: III
PROPER SHIPPING NAME: AMMONIUM HYDROXIDE

| 15. REGULATORY INFORMATION |  |
| --- | --- |

OSHA CATEGORY: NOT REGULATED

SARA 313 SUPPLIER NOTIFICATION: NOT REPORTABLE

ALL INGREDIENTS ARE LISTED ON THE TSCA INVENTORY:

| 16. OTHER INFORMATION |  |
| --- | --- |

SPECIAL PRECAUTIONS OR OTHER COMMENTS:
THE INFORMATION ACCUMULATED HEREIN IS BELIEVED TO BE ACCURATE BUT IS
NOT WARRANTED TO BE REGARDLESS OF WHOM IT ORIGINATES WITH. RECIPIENTS
ARE ADVISED TO CONFIRM PRIOR TO NEED THAT THE INFORMATION IS CURRENT,
APPLICABLE, AND SUITABLE TO THEIR CIRCUMSTANCES.